FlexNet-Treiberdokumentation    DG1SCR.EXE        (C)1994 Tom Sailer HB9JNX

Stand: 1.4.97

Ident: DG1SCR

Treiber fr das DG1SCR DSP Board

Dieser Treiber ermoeglicht es, das DG1SCR DSP Board als Packet-Modem zu

verwenden und zwar entweder fuer 1200 Baud AFSK oder fuer 9600 Baud FSK,

kompatibel zum G3RUH-Standard. Das Board wird an eine serielle Schnittstelle

angeschlossen. Es arbeitet als Modem und schickt einen Bitstrom zum PC, der

diesen auswerten muss (HDLC Protokoll decodieren), was Rechenleistung auf dem

PC kostet. Beim Einsatz als 9600 Baud-Modem muss unbedingt ein analoger Filter

am Ausgang des Boardes angeschlossen werden!

Da Juergen in neueren Monitorversionen Hardware-Handshake einsetzt,

kann RTS nicht mehr als PTT-Leitung verwendet werden!

Hardwarevoraussetzungen: Ausser Rechenleistung und einer Standard-seriellen

Schnittstelle (16550 wird unterstuetzt) keine.

Parameter:

/?,/h    Beischreibung der Parameter

/c       COM Nummer

/b,/p    Basisaddresse der Schnittstelle (falls /c nicht verwendet wird)

/i       Interrupt-Nummer (falls /c verwendet wird, muss /i nachher kommen)

/d       DTR invertieren (wird benoetigt, um den DSP zu initialisieren)

/r       RTS invertieren

/lo      Linke Ausgangsabschwaechung (0..95dB)

/ro      Rechte Ausgangsabschwaechung (0..95dB)

/o       Ausgangsabschwaechung beider Kanaele (0..95dB)

/lg      Linke Eingangsverstaerkung (0..23dB)

/rg      Rechte Eingangsverstaerkung (0..23dB)

/g       Eingangsverstaerkung beider Kanaele (0..23dB)

/m       Mic-Eingang benutzen

/l       Line-out abschalten

/he      Headphone-out abschalten

/s       Speaker-out abschalten

Das 9600 Baud-Modem ist sehr empfindlich auf DC-Pegel am Eingang. Man muss

also sicher gehen, dass das Empfaengersignal um die Masse zentriert ist!

Bei einigen RMNC-Karten ist das naemlich nicht der Fall.

Limitationen:

- Kalibration schaltet verzoegert aus (bis max. 45s bei 1200 Baud).

- Messung des TX-Delays anderer Stationen wird nicht unterstuetzt

- Das 9600Baud Modem braucht eine analoge Nachfilterung, da die Rechenleistung

des DSP's dafuer nicht mehr reicht

- Der 20dB Verstaerker des Mic-Einganges wird abgeschaltet. Dies ist allerdings

bei Codec Revisionen vor E nicht moeglich (die Codec Revision wird bei

Treiberstart angezeigt)

---------------------------------------------------------------------------

FlexNet driver documentation     DG1SCR.EXE       (C)1994 Tom Sailer HB9JNX

Date: 5.2.95

Ident: DG1SCR

Driver for use with the DG1SCR DSP board

This driver allows DG1SCR's DSP board to be used either as 1200 baud AFSK

modem or as 9600 baud FSK modem, compatible to the G3RUH standard. The

board works as a modem frontend, sending a serial bitstream to the PC, which

then must decode it (HDLC protocol), which requires some processing power on

the host PC. With 9600 baud, the analog output MUST be filtered with an analog

filter!

Starting with newer versions of Juergen's monitor EPROMs, RTS is used

for hardware handshake and can no longer be used as PTT signal

Minimal hardware configuration: A standard serial interface (16550 is

supported) and some processing power.

Parameter:

/?,/h    description of the parameters

/c       the COM port number

/b,/p    base address of the serial interface (if /c is not used)

/i       the IRQ number (if /c is used, /i must be placed afterwards)

/d       invert DTR (DTR serves as the reset line for the DSP)

/r       invert RTS

/lo      left output attenuation (0..95dB)

/ro      right output attenuation (0..95dB)

/o       output attenuation of both channels (0..95dB)

/lg      left input gain (0..23dB)

/rg      right input gain (0..23dB)

/g       input gain of both channels (0..23dB)

/m       use mic input

/l       switch off line out

/he      switch off headphone out

/s       switch off speaker out

The 9600 baud modem is very sensitive to DC levels at the input. Make sure that

the receiver signal is centered around ground!

Limitations:

- Calibration finishes with some delay (maximum about 45secs at 1200 Baud)

- Measurement of the TX delay of other stations is not supported

- The 9600 Baud modem requires an analog filter, because the computing power

of the DSP did not suffice for a better digital one

- The 20dB amp at the mic input is switched off. This is not possible on

pre revision E codecs. (the codec revision is displayed at the start of

the driver)

Autor:  Tom Sailer

Weinbergstrasse 76

CH-8408 Winterthur

Packet: HB9JNX @ HB9W.CHE.EU

EMail:  sailer@ips.id.ethz.ch