FlexNet-Treiberdokumentation    ETHER32.EXE           (C)1996 G. Jost DK7WJ

Stand: 15.09.96

Ident: ETHER32

Treiber f�r Win32-Socket-Protokolle IPX und IP

Unter Win95 mu� der Zugriff auf die Ethernet-Protokolle �ber das WINSOCK-

Interface erfolgen. Dieses ist jedoch erst nach dem Start von Windows

verf�gbar. Dieser Treiber wird als FlexNet-Kanaltreiber f�r die ben�tigte

Anzahl von Kan�len installiert und parametriert.

Nach dem Start der Windows-Oberfl�che mu� FLEXCTL.EXE aktiv sein. Es wird,

falls vorhanden, automatisch gestartet. Hier stehen noch weitere Features

zur Verf�gung.

Nur bei laufendem FLEXCTL.EXE werden die Ethernet-Frames behandelt!

Der Treiber darf nur einmal geladen werden, auch wenn mehrere Kan�le gebraucht

werden. Anderenfalls wird nur die erste geladene Instanz unterst�tzt.

Die Parametrierung der Kan�le erfolgt in der Kommandozeile.

Im Betrieb kann die Parametrierung (auch aus der Windows-DOS-Box) mit

ETH32CFG ge�ndert werden. Neue Kan�le k�nnen jedoch dann nicht mehr

hinzugef�gt werden. Die �nderungen werden nicht gespeichert, permanente

Zuordnungen m�ssen also im Start-Batchfile eingetragen sein.

Unter Windows m�ssen die verwendeten Protokolle installiert sein.

Also f�r AXIP der TCP/IP-Stack und/oder f�r AXIPX das IPX-Protokoll.

ACHTUNG: RAW-IP (AXIP) funktioniert noch nicht, also AXIP_UDP verwenden.

Die optionale Angabe der Portnummer bei AXIP setzt nur die Portnummer beim

Partner, empfangen wird immer auf Port 93!

Wie gewohnt laufen IP-Kan�le Punkt-zu-Punkt, w�hrend IPX Broadcasts verwendet.

Mit IPX kann also der explizite Eintrag der Ethernet-Partner unterbleiben.

-----------------------------------------------------------------------------

FlexNet driver documentation    ETHER32.EXE           (C)1996 G. Jost DK7WJ

Translation: Matthias Welwarsky, DG2FEF

Date: 03/31/97

Ident: ETHER32

>Under Win95, the WinSock interface has to be used to access the network

protocols. As FlexNet runs within the so called System-VM (and thus within

a DOS environment) it has no direct access to this interface. ETHER32

performs as a coupling device between FlexNet and the WinSock services.

The Win32 side of this coupling is done by a process called "FlexNet Control

Center", which is automatically launched after the 32bit kernel of Windows

has been booted. For this the FLEXCTL.EXE program must be installed into the

FlexNet directory, and the PATH environment variable must point to this

directory. The coupling will only work while the FLEXCTL.EXE is running.

WARNING: This applies only if the Win32 environment is running, i.e. if you

only boot DOS7 or run the computer in MS-DOS mode, FLEXCTL.EXE (which is a

Win32 application) cannot be started and ETHER32 will consequently fail!

Having this is mind the installation and configuration of ETHER32 is rather

easy: The driver needs to be installed like any other FlexNet L1 driver,

operating parameters are passed via the commandline. You can reconfigure

each ETHER32 channel with the ETH32CFG utility while FlexNet is running

(even from a DOS-Box under Windows), however you cannot change the channel

count.

The driver must only be loaded ONCE, even if more than one channel is

to be used. The driver will prevent itself from beeing activated in a second

instance. As the WinSock interface is used, the protocols requested through

ETHER32 need to be installed and properly configured. That is: You need to

install the TCP/IP services of Win95 if you want to use AXIP, and IPX if you

want to use IPX, respectively.

ATTENTION: AXIP_RAW (i.e. &quot;real&quot; AXIP) isn't supported yet, thus

for linking to other stations via AXIP you need to use UDP encapsulation

(AXIP_UDP). For UDP, the destination port number may be set (and reset

using ETH32CFG). The receiver port number is hardcoded to 93.

AXIP_RAW will be supported as soon as we find out how to utilize this feature

through WinSock (read: maybe never).